#### FOR INTERNAL PSI USE ONLY

Version 2.54 Release Notes

# 2 April 2008

Last Released Versions:

9016 V2.51

9816 V2.51

9021 V2.45

9022 V2.45

903x V2.40

9033 V2.48

9034 V2.53

9038 V2.53

9046 V2.50

# Version 2.54 Release includes the following Modules:

9034 V2.54

9038 V2.54

# Version 2.54 Changes

- Eliminates a bug that occurred in firmware version 2.53 for 9034 and 9038. The bug caused the following parameters to be set to the factory defaults at module startup. Automatic UDP Broadcast (default = OFF), IP Listen Port (default = 9000), TCP Back Off Delay (default = OFF), IP Address Mode (default = STATIC).